ALTERNATOR A.C.G. CHECK

(For Tracker, GTR 50, and Oliver)

ALTERNATOR A.C.G.

Set instrument range to 2000 Ω

Surge (coil resistance):	within 650 to 850 Ω; take red/black wire as positive and connect negative to body of A.C.G.

Pulsor (ignition resistance):	For Tracker within 80 to 120 Ω; take blue/yellow wire as positive and connect negative to body of A.C.G.

For GTR 50 within 80 to 120 Ω; take white/red wire as positive and connect negative to body of A.C.G.

For Oliver; within 290 to 320 Ω; take white/blue wire as positive and white/green as negative.

If not within range: 		Malfunction

REGULATOR CHECK

(For Tracker)

Set instrument range to 200 Ω

Take black wire as positive

Black to red:	2.68Ω

Black to pink:	2.14Ω

Black to yellow:	1.27Ω

Black to green:	.556Ω

Must have those values if not malfunction.