#### Fuel supply system

#### An electric pump, controlled by the ECU, feeds fuel to the injection manifold. The fuel is then fed at a pressure of around 7.5 bar, limited and regulated by a pressure regulator. The regulator is slaved to the compressor air pressure to continually maintain a differential pressure of 2.5 bar between the air (at around 5 bar) and the fuel. This makes the fuel delivery proportional to the injector opening time.
The pump functions for 3 seconds when the ignition is turned on in order to pressurise the fuel system.