#### Petrol pressure regulator

|  | #### Set pressure : 2.5 bars A : Fuel inlet B : Fuel return to tank C : Inlet pressure gauge Fuel pressure is limited by a valve that opens at 2.5 bars, enabling surplus fuel to return to the tank |
| --- | --- |
|  | #### Pressure regulation is slaved to inlet pressure. The fuel pressure regulator thus maintain a pressure difference of 2.5 bars between the injector inlet and outlet, independently of engine operating conditions. Hence it is the ECU that controls mixture richness via injection time. |