#### Synoptics

| Throttle unit; INJECTION /IGNITION ECU; Petrol injector. Engine speed and position sensor; Air injector. Engine temperature sensor; Ignition coil. Battery voltage; Fuel pump. Altitude sensor(option); Oil pump. Diagnostic plug; Diagnostic light. | 1. Fuel injector 2. Injection rail 3. Air injector 4. Pressurised air inlet; A Pressurised fuel (8 bar) B Mixture chamber C Combustion chamber D Pressurised air (5 bar). To convey the fuel, the system uses pressurised air (D). The air is pressurised by a compressor driven by the crankshaft. The pressurised fuel (A) is injected into the air injector mixture chamber (B). The pressurised air (D) is fed into the air injector mixture chamber (B), and mixes with the fuel injected. The air-fuel mixture is then injected directly into the combustion chamber (C) as the piston rises. |
| --- | --- |