EL - Specifikationer

| Rodoshow * BigMax * HOT50 * T-Rex50 |  |
| --- | --- |
| Source coil resistance | 555 ohm +/- 20% |
| Voltage | 1500 rpm - 292V |
|  | 2000 rpm - 296V |
|  | 5000 rpm - 222V |
| Ignition coil 1st resistance | 0,17 ohm |
| 2nd resistance | 3,6 K ohm |
| Pick up coil resistance | 120 ohm +/- 20% |
| Pick up rpm | 178 rpm Upper limit 300 rpm |
| Comet |  |
| Source oil resistance | 386 ohm +/- 20% |
| Voltage | 1500 rpm - 316V |
|  | 2000 rpm - 318V |
|  | 5000 rpm - 256V |
| Ignition coil 1st resistance | 0,17 ohm +/- 20% |
| 2nd resistance | 3,6 K ohm |
| Pick up coil resistance | 15,6 ohm +/- 20% |
| Pick up rpm | 240 rpm Upper limit 300 rpm |
| T-Rex150 |  |
| Source coil resistance | 800 ohm +/- 20% |
| Voltage | 1500 rpm - 308V |
|  | 2000 rpm - 328V |
|  | 5000 rpm - 256V |
| Ignition coil 1st resistance | 0,17 ohm |
| 2nd resistance | 3,6 K ohm |
| Pick up coil resistance | 120 ohm +/- 20% |
| Pick up rpm | 182 rpm Upper limit 300 rpm |